- 配置整机到主天线相位中心/NHC中心/用户自定义位置的杆臂值

- 格式：

- SETLEVERARM PARAM1 X Y Z

- 参数：

- PARAM1：目标杆臂（ANT1、NHC、USER）

- ANT1：整机到主天线相位中心杆臂

- NHC：整机到NHC约束中心杆臂（一般为后轮轴中心）

- USER：整机到用户自定义位置杆臂

- X/Y/Z：杆臂长度——按照车体坐标系轴向定义（X、Y、Z按照前右下顺序，X指向车前为正，Y指向车右为正，Z指向车下为正），从整机坐标系原点（见丝印）出发，测量出目标点相较于整机坐标系原点的位置。

- 单位：米（m）

- 范围：-100m~100m

- 示例：

- SETLEVERARM ANT1 0.55 -0.1 -1.14

- 配置惯导结果输出原点

- 格式：

- SETOUTPUTCENTER PARAM

- 参数：

- PARAM：惯导结果输出原点

- 可选如下参数：

- ANT1：惯导输出以天线相位中心为原点的结果

- NHC：惯导输出以NHC约束中心（一般为后轮轴）为原点的结果

- USER：惯导输出以用户自定义位置为原点的结果

- 默认输出以天线相位中心为原点结果

- 以上各参数对应原点位置与SETLEVERARM指令配置值匹配

- 示例：

- SETOUTPUTCENTER USER

- 配置整机坐标系相对于车体坐标系安装角

- 格式：

- SETROTATIONANGLE P1 P2 P3

- 参数：

- P1/P2/P3：坐标系三轴旋转角度

- 车辆坐标系按Z轴->Y轴->X轴顺序，遵循右手定则，分别旋转P3/P2/P1度后与整机坐标系重合。默认整机坐标系与车辆坐标系重合，角度为0。

- 单位：度（°）

- 范围：-180°~180°

- 示例：

- SETROTATIONANGLE 0 180 0

- 配置主天线到从天线连线矢量与车辆坐标系x轴在水平面夹角

- 格式：

- SETINSHEADINGDIFF PARAM

- 参数：

- PARAM：双天线安装角

- 俯视车辆，主线天到从天线连线矢量与车辆坐标系x轴在水平面的夹角，顺时针为正。默认主天线在后，从天线在前，双天线安装角为0。

- 单位：度（°）

- 范围：-180°~180°

- 示例：

- SETINSHEADINGDIFF 90

- 配置载体类型

- 格式：

- SETCARRIERMODEL PARAM

- 参数：

- PARAM：载体类型

- 可选如下参数：

- 0：乘用车

- 1：低速车载

- 默认为乘用车模型

- 示例：

- SETCARRIERMODEL 0

- 配置固定解初始化开关

- 格式：

- SETRTKINIT PARAM

- 参数：

- PARAM：开关参数

- 可选如下参数：

- ON：开启固定解初始化（默认）

- OFF：关闭固定解初始化

- 默认开启

- 示例：

- SETRTKINIT ON

- 配置惯导初始化速度门限

- 格式：

- SETALIGNVEL PARAM

- 参数：

- PARAM：初始化速度

- 无双天线且未开启轨迹初始化功能时，载体完成组合导航初始化需要保证的最低行驶速度。默认初始化速度3.0m/s

- 单位：米/秒（m/s）

- 范围：≥1m/s

- 示例：

- SETRTKINIT ON

- 配置GNSS拒止环境下，纯惯导递推时间阈值

- 格式：

- SETDRTIMEOUT PARAM

- 参数：

- PARAM：最大惯导递推时长

- 默认值为7200

- 单位：秒（s）

- 示例：

- SETDRTIMEOUT 2700

- 配置惯导初始化前结果输出开关

- 格式：

- SETALIGNINGOUTPUT PARAM

- 参数：

- PARAM：初始化前结果输出开关

- 可选如下参数：

- ON：开启初始化前结果输出

- OFF：关闭初始化前结果输出

- 默认开启

- 示例：

- SETALIGNINGOUTPUT ON

- Log指令

- 格式：

- Log <port> <message-type> <type-trigger> <period>

- 参数：

| - 参数 | - 描述 | - 数值 | - 默认值 | - 单位 |
| --- | --- | --- | --- | --- |
| - port | - 端口 | - com1/com2 | - 发送指令的端口 |  |
| - message-type | - 输出报文类型 | - NEMA报文、BESTPOS、INSPVA、INSPVAX |  |  |
| - type-trigger | - 报文触发类型 | - ONTIME/ONCE | - ONCE |  |
| - period | - 输出周期 | - 1/0.5/0.1/0.01 |  |  |
| - 说明：惯导相关指令仅com2系统口支持，com1口同K922 |  |  |  |  |

- INSPVA：

#INSPVAA,USB1,0,67.5,FINESTEERING,2209,490558.000,02000020,18bc,16809;2209,490558.000000000,51.15043714042,-114.03067871718,1080.3548,0.0051,-0.0014,-0.0012,-0.296402993,0.311887972,157.992156267,INS_SOLUTION_GOOD*cc698020

| Field | Field Type | Description | Format | Binary Bytes | Binary Offset |
| --- | --- | --- | --- | --- | --- |
| 1 | Log header | INSPVA header For information about log headers, see ASCII, Abbreviated ASCII or Binary. | - | H | 0 |
| 2 | Week | GNSS Week | Ulong | 4 | H |
| 3 | Seconds | Seconds from week start | Double | 8 | H+4 |
| 4 | Latitude | Latitude [degrees] | Double | 8 | H+12 |
| 5 | Longitude | Longitude [degrees] | Double | 8 | H+20 |
| 6 | Height | Ellipsoidal Height [m] | Double | 8 | H+28 |
| 7 | North Velocity | Velocity in a northerly direction (a negative value implies a southerly direction) [m/s] | Double | 8 | H+36 |
| 8 | East Velocity | Velocity in an easterly direction (a negative value implies a westerly direction) [m/s] | Double | 8 | H+44 |
| 9 | Up Velocity | Velocity in an up direction [m/s] | Double | 8 | H+52 |
| 10 | Roll | Right-handed rotation from local level around y-axis in degrees | Double | 8 | H+60 |
| 11 | Pitch | Right-handed rotation from local level around x-axis in degrees | Double | 8 | H+68 |
| 12 | Azimuth | Left-handed rotation around z-axis in degrees clockwise from North This is the inertial azimuth calculated from the IMU gyros and the SPAN filters. | Double | 8 | H+76 |
| 13 | Status | INS Status, see Table: Inertial Solution Status | Enum | 4 | H+84 |
| 14 | xxxx | 32-bit CRC | Hex | 4 | H+88 |
| 15 | [CR][LF] | Sentence Terminator (ASCII only) | - | - | - |

- INSPVAXA:

#INSPVAXA,USB1,0,65.5,FINESTEERING,2209,490782.000,02000020,46eb,16809;INS_SOLUTION_GOOD,INS_PPP,51.15043710672,-114.03067871892,1097.3598,-17.0001,-0.0016,0.0002,0.0029,-0.308665944,0.297893298,157.960833016,0.1816,0.1816,0.1808,0.0018,0.0018,0.0016,0.0292,0.0292,0.0582,13000045,0*a74644a2

| Field | Field Type | Data Description | Format | Binary Bytes | Binary Offset |
| --- | --- | --- | --- | --- | --- |
| 1 | Log header | INSPVAX header For information about log headers, see ASCII, Abbreviated ASCII or Binary. |  | H | 0 |
| 2 | INS Status | Solution status See Table: Inertial Solution Status | Enum | 4 | H |
| 3 | Pos Type | Position type See Table: Position or Velocity Type | Enum | 4 | H+4 |
| 4 | Lat | Latitude (degrees) | Double | 8 | H+8 |
| 5 | Long | Longitude (degrees) | Double | 8 | H+16 |
| 6 | Height | Height above mean sea level (m) | Double | 8 | H+24 |
| 7 | Undulation | Undulation (m) | Float | 4 | H+32 |
| 8 | North Vel | North velocity (m/s) | Double | 8 | H+36 |
| 9 | East Vel | East velocity (m/s) | Double | 8 | H+44 |
| 10 | Up Vel | Up velocity (m/s) | Double | 8 | H+52 |
| 11 | Roll | Roll in Local Level (degrees) | Double | 8 | H+60 |
| 12 | Pitch | Pitch in Local Level (degrees) | Double | 8 | H+68 |
| 13 | Azimuth | Azimuth in Local Level (degrees) This is the inertial azimuth calculated from the IMU gyros and the SPAN filters. | Double | 8 | H+76 |
| 14 | Lat σ | Latitude standard deviation (m) | Float | 4 | H+84 |
| 15 | Long σ | Longitude standard deviation (m) | Float | 4 | H+88 |
| 16 | Height σ | Height standard deviation (m) | Float | 4 | H+92 |
| 17 | North Vel σ | North velocity standard deviation (m/s) | Float | 4 | H+96 |
| 18 | East Vel σ | East velocity standard deviation (m/s) | Float | 4 | H+100 |
| 19 | Up Vel σ | Up velocity standard deviation (m/s) | Float | 4 | H+104 |
| 20 | Roll σ | Roll standard deviation (degrees) | Float | 4 | H+108 |
| 21 | Pitch σ | Pitch standard deviation (degrees) | Float | 4 | H+112 |
| 22 | Azimuth σ | Azimuth standard deviation (degrees) | Float | 4 | H+116 |
| 23 | Ext sol stat | Extended solution status See Table: Extended Solution Status | Hex | 4 | H+120 |
| 24 | Time Since Update | Elapsed time since the last ZUPT or position update (seconds) | Ushort | 2 | H+124 |
| 25 | xxxx | 32-bit CRC (ASCII and Binary only) | Hex | 4 | H+126 |
| 26 | [CR][LF] | Sentence terminator (ASCII only) | - | - | - |

- 示例：

- LOG INSPVAA ONTIME 1

- LOG INSPVAXA ONTIME 1